# PHSX 536 Final Project Proposal: Voltage Controlled Filter

Grant Saggars

Voltage controlled filters are a classic component in audio engineering, and were originally created in the late 20th century. They provide ways to shape a simple waveform, such as a square or triangle wave, into anything from a classic rock guitar sound to a kick drum. I aim to implement an op-amp based design, popularized in Moog synthesizers during the late 20th century. This design deviates from the classic ladder filter by incorporating a diode ladder to maintain a more uniform voltage-dependent control.

Circuit Schematic in Falstad

Parts List

Semiconductors

- 11x 1N4148 diode

Capacitors

- 2x 1uF film capacitor (or 2x 0.5uF)

- 5x 1nF film capacitor (or 2x 0.5uF)

ICs

- 7x TL074

Resistors

- 2x 200k resistor

- 2x 100k resistor

- 1x 47k resistor

- 8x 33k resistor

- 2x 10k resistor

- 1x 2k7 resistor

- 2x 2k resistor

- 1x 1k resistor

- 1x 330 resistor

Potentiometers

- 2x 100k linear mono potentiometer

Misc

- 3x audio jack socket